ESCUELA SUPERIOR POLITECNICA DEL LITORAL

FACULTAD DE CIENCIAS NATURALES Y MATEMATICAS

FIRST EXAMINATION FOR WATER TREATMENT COURSE

NAME:________________________________________________________DATE: July 2, 2019

60 POINTS TEST

Questions 1-10 :  2 points each

- What of the following is more likely to be found in groundwater?

- Natural organic matter

- Minerals

- Coliform bacteria

- Suspended solids

- What type of materials should be removed in the raw water intake?

- Dissolved solids

- Turbidity

- Bacteria

- Hardness

- What causes the water to look cloudy?

- Dissolved solids

- Bicarbonates

- Hardness

- Suspended material

- What does the addition of carbon dioxide do to the pH of water?

- Raises pH

- Lowers pH

- Unaffected by it

- Creates instability of pH

- In the analysis of alkalinity, the concentration of acid is:

- 0,1 M

- 0,2 M

- 0,02 N

- 1 M

- Which of the following compounds better describes the non-carbonaceous hardness:

- Calcium bicarbonate

- Sodium carbonate

- Calcium sulfate

- Carbon dioxide

- If a sample of water has a pH=10, What is the value of alkalinity due to OH- ions?

- 4,0 mg/l CaCO3

- 10-10 mg/l CaCO3

- 1,5 x 10-4 mg/l CaCO3

- 5 mg/l CaCO3

- What causes hardness in water?

- Manganese and chloride

- Magnesium and calcium

- Manganese and potassium

- Iron and lead

- The process of removing dissolved solids from water using membranes is called:

- Lime and soda ash softening

- Ion exchange process

- Reverse osmosis

- Microfiltration

- The process of removing carbon dioxide from water is called:

- Filtration

- Aeration

- Decarbonation

- Recarbonation

- (5 points) What is the alkalinity of water?.  What are the chemical species that contribute to water alkalinity and what is the relative importance of each species?

- (5 points) A sample of water has a Ca++ content of 250 mg/l and 150 mg/l of MgCO3.    Calculate the hardness of this water and its alkalinity.

- (10 points) Explain briefly what oxidation pond is and when it is commonly used in wastewater treatment.  What type of process takes place in it?

- (10 points) A water analysis shows the following results:

Ca+2	65 mg/l			HCO3-     183 mg/l

Mg+2     35 mg/l			SO4-2       120 mg/l

Na+       46 mg/l			Cl-           71 mg/l

K+        14,04 mg/l		PO4-3      84,55 mg/l

Calculate the ionic balance of this water.  What is the total hardness and the total alkalinity of this water?.  From the results of hardness and alkalinity determine what type of water is.  Explain your answer.

- (10 points) A water has the following characteristics:

Ca++  = 40 mg/L		HCO3- (Alc) = 135 mg/L

Mg++= 14,7 mg/L	SO4-2 = 29 mg/L

Na+ =  13,7 mg/L	Cl- = 17,8 mg/L

Make the bar graph and list the hypothetical combinations for this water.