| YEAR: 2019 | TERM: 1st Term |
| --- | --- |
| SUBJECT: Chemical Process Design | PROFESSOR: Santiago D. Salas, Ph.D. |
| EVALUATION: Final Exam (mejoramiento) |  |
| TIME LENGTH: 2 hours (14h00 – 16h00) | DATE: September 13th, 2019 |

HONOR COMMITMENT

I, ………………………………………………………………………………………………………………..…………… by signing this commitment, I acknowledge that the present exam is designed to be solved individually, that I can use an ordinary calculator for arithmetic calculations, a pen or pencil; that I can only communicate with the person responsible for taking the exam; and, any communication instrument that I have brought, I must turn it off and deposit it in the front of the room, along with any other material that is attached to it. I should also not consult any books, notes, or additional handouts to those that are delivered in this evaluation. The topics I must develop in an orderly manner.

I sign at the bottom of this commitment, as proof of having read and accepted the previous statement.

"As a student of ESPOL I commit myself to fight mediocrity and act honestly, that's why I do not copy or let copy."

Signature: __________________                          Registration Number: __________________                         Class: ________________

INSTRUCTIONS:

- You are allowed to use an English – Spanish or English – English dictionary.

- The exam is worth a total of 100 points.

- Allocate your time wisely. Use the number of points assigned to each problem as your guide.

- In order to get full credit on the problems, you must show ALL your work!

- MULTIPLE CHOICE (40 points):

Choose the best answer and justify your selection for each of the following questions. The questions are worth four (4) points each.

  - A typical process flow diagram contains:

- Basic control loops.

- Utility streams.

- All process flow streams identified by a number.

- All of the above.

- None of the above.

Justification: ________________________________________________________________

  - In the block flow diagram:

- The flow goes from left to right whenever is possible.

- The operations are shown by blocks.

- The major flow lines are shown with arrows to give direction of flow.

- All of the above.

- None of the above.

Justification: ________________________________________________________________

  - The piping and instrumentation diagram:

- Is a guide for those who will be responsible for the final design and construction.

- Includes every mechanical aspect of the plant.

- Is also known as the mechanical flow diagram.

- All of the above.

- None of the above.

Justification: ________________________________________________________________

  - The most useful process diagram for chemical engineers is the:

- Piping and instrumentation diagram.

- Process block diagram.

- Process flow diagram.

- All of the above.

- None of the above.

Justification: ________________________________________________________________

  - Which separation method should be chosen as first option when purity of both products is required?

- Reverse osmosis.

- Leaching.

- Evaporation.

- All of the above.

- None of the above.

Justification: ________________________________________________________________

  - A commodity chemicals are:

- Those produced by companies in small quantities usually in batch processes.

- Produced in large quantities usually in continuous processes.

- Consider the size and flexibility of the company.

- All of the above.

- None of the above.

Justification: ________________________________________________________________

  - The front end engineering design (FEED) is also known as:

- The feasibility design.

- The design for construction.

- The detailed design.

- All of the above.

- None of the above.

Justification: ________________________________________________________________

  - The facility layout definition is:

- The size and flexibility of a chemical plant.

- Minimum movement of people, material and resources.

- An arrangement of everything needed for the production of goods or delivery of services.

- All of the above.

- None of the above.

Justification: ________________________________________________________________

  - Flexibility is needed in a plant layout mainly because of:

- Security.

- The types of feedstock materials.

- Future changes.

- All of the above.

- None of the above.

Justification: ________________________________________________________________

  - In the LOPA analysis what is the second protection layer in a chemical plant?

- The chemical plant design.

- The process control system.

- The prevention system.

- All of the above.

- None of the above.

Justification: ________________________________________________________________

- True or False (10 points):

Selection. Two (2) points each.

2.1 For adequately designing compact heat exchangers it is mandatory to follow the TEMA standards. (T / F)

2.2 McCabe-Thiele and Ponchon-Savarit are simple methods to calculate distillation columns in binary systems. (T / F)

2.3 In catalytic reactions we find external diffusion, surface adsorption, surface desorption and diffusion of products. (T / F)

2.4 To manage heat effects in chemical reactors generates run away reactions. (T / F)

2.5 To achieve an appropriate plant layout it is important to do not include the business strategic plant and tactical objectives. (T / F)

- PROBLEMS. Answer each of the questions below completely. You must show ALL your work to get full credit.

You have been hired by SDQ Oil Corp. for designing a horizontal three-phase inlet separator for ITT oil field. The input information given to you is the following:

Qo = 33 m3/hr, Qw = 19.8 m3/hr, Qg = 5 902 s m3/hr, P = 690 kPa, T = 32.20 ˚C, Sg = 0.6, (SG)w = 1.07

SGo = 30 ˚API, Z = 0.99, µo = 10 cp, µw = 1 cp.

Droplet removal = 100 microns liquid, 500 microns water, 200 microns oil

Retention time for oil and water = 10 min

3.1 Sketch a diagram of the vessel and compute the critical gas velocity (5 points).

3.2 Estimate the diameter and volume of the three-phase horizontal separator (10 points).

3.3 Estimate the cost of the two phase horizontal inlet separator (10 points).

3.4 “Vinos doña Carmen S.A.” is a company that distributes wine in the coast of Ecuador. It has four main clients Hacienda San Luis, Estadio Banco Pacífico, Supermaxi and Mi Comisariato. Due to expected increase in demand, Vinos doña Carmen S.A. wants to add one more plant. Two alternative locations are under consideration: Machala and Manta. The table below shows the capacity of the existing and proposed plants, demands and transportation costs per unit from a plant to the distributor.

Apply the least cost method or the Vogel approximation to obtain the best solution of each possibility, provide the final cost and units to be distributed per client (10 points per scenario). After your analysis, provide the place where the pant should be built and why your method is reliable (5 points).

|  | Hac. San Luis | Estadio B. Pacíf. | Supermaxi | Mi Comisariato | Capacity, bottles |  |
| --- | --- | --- | --- | --- | --- | --- |
| Guayaquil | 1 | 2 | 3.75 | 3.5 | 3000 | Supply |
| Portoviejo | 2.75 | 1.25 | 3 | 2.25 | 2000 |  |
| Milagro | 4.25 | 3.25 | 1.5 | 1.75 | 1500 |  |
| Machala (p) | 3 | 3.75 | 4.75 | 2.25 | 1000 |  |
| Manta (p) | 4.75 | 2.75 | 2.25 | 2.75 | 1000 |  |
| Demand (20%) | 2080 | 1500 | 3100 | 820 |  |  |
|  | Demand |  |  |  |  |  |

Additional Information:

vc=Kρl-ρgρg

ºAPI=141.5S.G.-131.5

ρg=3.492Sg∙P[kPa]T[K]∙Z

PV=nRTZ

Standard conditions: 60 ºF, 1 atm.

1 atm=14.7 psi=1.01325 bar=101.325 kPa

For gas capacity:		Lss=Leff+D

For liquid capacity:		Lss=43Leff

For cost estimation:

log10Cp'0=K1+K2log10A+K3log10A2

Fp, vessel=P+1D2[850-0.6(P+1)]+0.003150.0063

If Fp, vessel is less than 1 (corresponding to tvessel < 0.0063 m), then Fp, vessel=1. For pressures less than 0.5 barg, Fp, vessel=1.25.

CBM=Cp0FBM=Cp'0B1+B2FMFP

FM=2.8 for stainless steel (SS); 2.75 for Cu